INTRODUCTION

1.PREFACE

This manual gives you stepwise intructions on installation,usage and

trouble shooting of the mouse. The content of this manual applies to

Microsoft,Mouse System & PS/2 mouse users

In order to use this manual, we assume users have basic understanding

in personal computer applications plus basic DOS commands.

Please check first the mouse you acquired to be serial mouse or PS/2

mouse, since they belong to different of computers.

2.EXPLANATION OF MOUSE DRIVER FILES

MOUSE.COM   -  The file servers to sctivate the mouse, By typing the

MOUSE command, the driver program will be loaded into

computer memory.

MOUSE.SYS   -  Add "decice=mouse.sys" in your config.sys file .this

will enable the automatic loading of the mouse.sys

into your computer memory whenever you start up your

pc.

TEST.EXE    -  A mouse test program, including setup resolution and

use left hand or right hand.

HOW TO INSTALL THE MOUSE

1.Put the mouse's ball into its housing by fllowing the procedures below:

(1) Turn the mouse upside down and turn the ball cover toward the

arrows' direction to open it.

(2) Put in the ball that goes with the mouse. Being careful, not

to drop the ball on your desk or floor.

(3) Close the ball cover by turning it reversing the arrows' direction.

2.Gently plug-in the mouse connector to one of your computer RS-232 port

or PS2 mouse port.

3 Please copy all files on the mouse driver diskette onto your current

path,then you may activate your mouse driver by any of the following

two ways:

(1) Type "mouse" and press the [enter] key. this will load the

"mouse.com" onto your computer memory.

1. for the detailed options of how the mouse driver can

be loaded, please type "mouse/? " for more information.

(2) Add "device = mouse.sys " in your config.sys files.

4.Mode selection:

Microsoft Mode    - simply power on the computer

Mouse System Mode - befor turning on the computer, press any one of

button until the computer has completed all power

up procedures.

* note : If you found a slide switch,marked " M " (2) & " P " (3) on

your mouse,please operate it in reference to the instruntion

as below:

" M " ------- the mouse works under Microsoft Mode.

" P " ------- the mouse works under Mouse System Mode.

PS/2 P.D.P Mode   - Simply power on the computer.

TROUBLE SHOOTING

1.Mouse dirty :

The rolling ball will accumulate dust that surely effects

fountionality, periodical cleaning of the ball with tap water and

soap is quite necessary.

(1) Turn off your computer and turn the upside down.

(2) Remove the ball housing cover.Clean the ball and case.

(3) Re-insert the ball and plug the mouse into the computer.

2.Mouse not working :

If the mouse is not working, the following steps of trouble shooting

are recommended.

(1) Check cable connections --- Be sure your mouse is connected to

COM 1 or COM 2.

(2) Check if driver loaded  --- Follow steps advised prexiously to

install driver.

(3) Check application software specific requirement ---

Some application software, i.e. autoCAD & Windows 3.X, has their

COM port and mouse mode decided when set-up, Users have to enter

into same mode and COM port to ensure mouse works correctly with

the application softwares